ESP8266 modul

- ESP8266 koneksi ke LCD I2C

- Wiring skema berikut:

- pastikan polaritas tegangan tidak terbalik antara 5v dan GND, jika sudah download

library lcd i2c disini.

Link download library: https://github.com/marcoschwartz/LiquidCrystal_I2C/archive/master.zip

- ekstrak library yang sudah di download dan rename folder

LiquidCrystal_I2C-master menjadi LiquidCrystal_I2C

- jika sudah, copy folder tersebut ke Document > Arduino > Libraries

- buka software arduino dan tulis program seperti berikut:

#include <LiquidCrystal_I2C.h>

// set the LCD number of columns and rows

int lcdColumns = 16;

int lcdRows = 2;

// set LCD address, number of columns and rows

// if you don't know your display address, run an I2C scanner sketch

LiquidCrystal_I2C lcd(0x27, lcdColumns, lcdRows);

void setup(){

// initialize LCD

lcd.init();

// turn on LCD backlight

lcd.backlight();

}

void loop(){

// set cursor to first column, first row

lcd.setCursor(0, 0);

// print message

lcd.print("Hello, World!");

delay(1000);

// clears the display to print new message

lcd.clear();

// set cursor to first column, second row

lcd.setCursor(0,1);

lcd.print("Hello, World!");

delay(1000);

lcd.clear();

}

- Hubungkan kabel USB dengan esp8266 dan upload program.

2. Sensor ultrasonic

- Wiring diagram sebagai berikut:

- pastikan polaritas tegangan 5V dan GND tidak terbalik

- buka software arduino dan tulis program berikut ini

const int trigPin = 12;

const int echoPin = 14;

//define sound velocity in cm/uS

#define SOUND_VELOCITY 0.034

#define CM_TO_INCH 0.393701

long duration;

float distanceCm;

float distanceInch;

void setup() {

Serial.begin(115200); // Starts the serial communication

pinMode(trigPin, OUTPUT); // Sets the trigPin as an Output

pinMode(echoPin, INPUT); // Sets the echoPin as an Input

}

void loop() {

// Clears the trigPin

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

// Sets the trigPin on HIGH state for 10 micro seconds

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

// Reads the echoPin, returns the sound wave travel time in microseconds

duration = pulseIn(echoPin, HIGH);

// Calculate the distance

distanceCm = duration * SOUND_VELOCITY/2;

// Convert to inches

distanceInch = distanceCm * CM_TO_INCH;

// Prints the distance on the Serial Monitor

Serial.print("Distance (cm): ");

Serial.println(distanceCm);

Serial.print("Distance (inch): ");

Serial.println(distanceInch);

delay(1000);

}

- Upload program dan buka serial monitor, hasil akan seperti dibawah ini.

3. Sensor DHT 11

- Buat skema kabel sebagai berikut:

- jika sudah, buka software arduino dan install library dengan cara klik Sketch >

include library > manage library > search DHT sensor library

- install juga library adafruit unified sensor

Setelah itu restart arduino program.

- Tulis program arduino sebagai berikut

#include "DHT.h"

#define DHTPIN 5 // pin GPIO 5

#define DHTTYPE DHT11 // DHT 11

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(9600);

Serial.println("DHTxx test!");

dht.begin();

}

void loop() {

delay(2000);

float h = dht.readHumidity();

float t = dht.readTemperature();

float f = dht.readTemperature(true);

if (isnan(h) || isnan(t) || isnan(f)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

float hif = dht.computeHeatIndex(f, h);

float hic = dht.computeHeatIndex(t, h, false);

Serial.print(F("Humidity: "));

Serial.print(h);

Serial.print(F("% Temperature: "));

Serial.print(t);

Serial.print(F("°C "));

Serial.print(f);

Serial.print(F("°F Heat index: "));

Serial.print(hic);

Serial.print(F("°C "));

Serial.print(hif);

Serial.println(F("°F"));

}

e. hubungkan esp dengan kabel usb dan tekan upload pada arduino, setelah selesai

buka serial monitor dan hasilnya akan seperti dibawah ini

4. Servo motor

- Buat rangkaian kabel sebagai berikut

- Download library servo disini: https://github.com/arduino-libraries/Servo. Setelah itu install library servo di folder arduino.

- Buat program ini dan upload.

#include <Servo.h> // servo library

Servo s1;

void setup()

{

s1.attach(0);  // servo attach D3 pin of arduino

}

void loop()

{

s1.write(0);

delay(1000);

s1.write(90);

delay(1000);

s1.write(180);

delay(1000);

}

5. PIR sensor

- Buat skema kabel sebagai berikut:

esp 8266=>PIR sensor

D6 	= LED

D7 	= data

3V3 	= +

GND	= -

- Buat program sebagai berikut

int Status = 12;  // Digital pin D6

int sensor = 13;  // Digital pin D7

void setup() {

pinMode(sensor, INPUT);   // declare sensor as input

pinMode(Status, OUTPUT);  // declare LED as output

}

void loop() {

long state = digitalRead(sensor);

if(state == HIGH) {

digitalWrite (Status, HIGH);

Serial.println("Motion detected!");

delay(1000);

}

else {

digitalWrite (Status, LOW);

Serial.println("Motion absent!");

delay(1000);

}

}

- Upload program.

6. BMP180 kontrol dengan esp8266

- Buat sambungan kabel sebagai berikut.

- Install BMP library: buka Sketch > Include Libraries > Manage Libraries.

- Tulisa program dan upload:

#include <Wire.h>

#include <Adafruit_BMP085.h>

#define seaLevelPressure_hPa 1013.25

Adafruit_BMP085 bmp;

void setup() {

Serial.begin(115200);

if (!bmp.begin()) {

Serial.println("BMP180 Not Found. CHECK CIRCUIT!");

while (1) {}

}

}

void loop() {

Serial.print("Temperature = ");

Serial.print(bmp.readTemperature());

Serial.println(" *C");

Serial.print("Pressure = ");

Serial.print(bmp.readPressure());

Serial.println(" Pa");

Serial.print("Pressure at sealevel (calculated) = ");

Serial.print(bmp.readSealevelPressure());

Serial.println(" Pa");

Serial.print("Altitude = ");

Serial.print(bmp.readAltitude());

Serial.println(" meters");

Serial.print("Real altitude = ");

Serial.print(bmp.readAltitude(seaLevelPressure_hPa * 100));

Serial.println(" meters");

Serial.println();

delay(1000);

}

c. Buka monitor arduino.

7.  Kontrol led melalui wifi

- Rangkai kabel sebagai berikut:

D4(GPIO2) → anode led

GND → GND

- Install library:

  - ESP8266WiFi.h, : https://github.com/esp8266/Arduino/tree/master/libraries/ESP8266WiFi

- Tulis program dan upload:

#include <ESP8266WiFi.h>

#include <WiFiClient.h>

//ESP Web Server Library to host a web page

#include <ESP8266WebServer.h>

//---------------------------------------------------------------

//Our HTML webpage contents in program memory

const char MAIN_page[] PROGMEM = R"=====(

<!DOCTYPE html>

<html>

<body>

<center>

<h1>WiFi LED on off demo: 1</h1><br>

Ciclk to turn <a href="ledOn">LED ON</a><br>

Ciclk to turn <a href="ledOff">LED OFF</a><br>

<hr>

<a href="https://circuits4you.com">circuits4you.com</a>

</center>

</body>

</html>

)=====";

//---------------------------------------------------------------

//On board LED Connected to GPIO2

#define LED 2

//SSID and Password of your WiFi router

const char* ssid = "circuits4you.com"; —--> ssid wifi anda

const char* password = "123456789";----> password wifi anda

//Declare a global object variable from the ESP8266WebServer class.

ESP8266WebServer server(80); //Server on port 80

//===============================================================

// This routine is executed when you open its IP in browser

//===============================================================

void handleRoot() {

Serial.println("You called root page");

String s = MAIN_page; //Read HTML contents

server.send(200, "text/html", s); //Send web page

}

void handleLEDon() {

Serial.println("LED on page");

digitalWrite(LED,LOW); //LED is connected in reverse

server.send(200, "text/html", "LED is ON"); //Send ADC value only to client ajax request

}

void handleLEDoff() {

Serial.println("LED off page");

digitalWrite(LED,HIGH); //LED off

server.send(200, "text/html", "LED is OFF"); //Send ADC value only to client ajax request

}

//==============================================================

//                  SETUP

//==============================================================

void setup(void){

Serial.begin(115200);

WiFi.begin(ssid, password);     //Connect to your WiFi router

Serial.println("");

//Onboard LED port Direction output

pinMode(LED,OUTPUT);

//Power on LED state off

digitalWrite(LED,HIGH);

// Wait for connection

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

//If connection successful show IP address in serial monitor

Serial.println("");

Serial.print("Connected to ");

Serial.println(ssid);

Serial.print("IP address: ");

Serial.println(WiFi.localIP());  //IP address assigned to your ESP

server.on("/", handleRoot);      //Which routine to handle at root location. This is display page

server.on("/ledOn", handleLEDon); //as Per  <a href="ledOn">, Subroutine to be called

server.on("/ledOff", handleLEDoff);

server.begin();                  //Start server

Serial.println("HTTP server started");

}

//==============================================================

//                     LOOP

//==============================================================

void loop(void){

server.handleClient();          //Handle client requests

}

- Cek di monitor akan keluar IP dan buka melalui HP atau PC pada jaringan anda.

Kombinasikan sensor dan aktuator servo dengan menggunakan komunikasi lewat WIFI sebagai latihan. Apabila ada pertanyaan dapat ditanyakan melalui email:mrutu.iot@gmail.com.